|  | Simulation of GIRPPRE_02 Date: Friday, March 28, 2025 Designer: Solidworks Study name: Static 1 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 4 Connector Definitions 5 Contact Information 5 Mesh information 5 Sensor Details 6 Resultant Forces 6 Beams 6 Study Results 7 Conclusion 9. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: GIRPPRE_02 Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Boss-Extrude1; Solid Body; Mass:0.00012905 kg Volume:8.06617e-07 m^3 Density:159.99 kg/m^3 Weight:0.00126469 N; D:\Mustafa\Solid work\Robot V2\Arm\GIRPPRE_02.SLDPRT Mar 4 23:35:42 2025.

# Study Properties Study name; Static 1. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; Automatic. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (D:\Mustafa\Solid work\Robot V2\Arm).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; Balsa. Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 2e+07 N/m^2. Elastic modulus:; 3e+09 N/m^2. Poisson's ratio:; 0.29. Mass density:; 159.99 kg/m^3. Shear modulus:; 3e+08 N/m^2.; SolidBody 1(Boss-Extrude1)(GIRPPRE_02). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); -6.55651e-07; 1; -2.12458e-08; 1. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Force-1; Entities:; 1 face(s), 1 plane(s). Reference:; Top Plane. Type:; Apply force. Values:; ---, ---, -1 N..

# Connector Definitions No Data

# Contact Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Blended curvature-based mesh. Jacobian points for High quality mesh; 16 Points. Maximum element size; 0.931305 mm. Minimum element size; 0.0465653 mm. Mesh Quality; High. ## Mesh information - Details Total Nodes; 13655. Total Elements; 8432. Maximum Aspect Ratio; 3.2822. % of elements with Aspect Ratio < 3; 100. Percentage of elements with Aspect Ratio > 10; 0. Percentage of distorted elements; 0. Time to complete mesh(hh;mm;ss):; 00:00:02. Computer name:; MOSTAFA_LABIB.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -6.55651e-07; 1; -2.12458e-08; 1. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

## Free body forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -1.63971e-07; 4.09782e-08; 8.78923e-09; 1.69242e-07. ## Free body moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 1e-33.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 2.233e+03N/m^2 Node: 547; 1.923e+06N/m^2 Node: 478. GIRPPRE_02-Static 1-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00mm Node: 9; 7.214e-02mm Node: 226. GIRPPRE_02-Static 1-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 2.359e-06 Element: 7154; 4.727e-04 Element: 7174. GIRPPRE_02-Static 1-Strain-Strain1.

# Conclusion